GatewayV2

PRODUCT TECHNICAL SPECIFICATION

| DOCUMENT ID | GWV2-PTS-001 |
| --- | --- |
| REVISION | 1.1 draft |
| STATUS | Draft for product and system-integration review |
| COMPANY | SynaptiX Technology Jsc |
| CLASSIFICATION | Internal Engineering |
| APPLICABLE FIRMWARE | Current GatewayV2 development baseline |
| APPLICABLE HARDWARE | Current GatewayV2 schematic/PCB baseline; released revision TBD |
| DATE | 14 August 2026 |

INTERNAL ENGINEERING

SynaptiX Technology Jsc  |  Industrial IoT Gateway

# Document Control

| CONTROL ITEM | VALUE |
| --- | --- |
| Document ID | GWV2-PTS-001 |
| Document type | Product Technical Specification |
| Company | SynaptiX Technology Jsc |
| Product | SynaptiX GatewayV2 |
| Revision | 1.1 draft |
| Audience | Product, system integration, firmware, hardware and QA |
| Classification | Internal Engineering |
| Applicable firmware | Current GatewayV2 development baseline |
| Hardware basis | Current GatewayV2 schematic/PCB baseline; released revision TBD |
| Primary hostname | synaptix-gateway.local |

| RULE | Product capabilities and released characteristics shall be approved at product level. Values marked TBD, Current baseline, or Engineering target shall not be treated as final released specifications until verified and approved. Schematic-derived protection components do not constitute EMC, safety, surge or environmental certification. |
| --- | --- |

# References and Approval

## Reference Documents

| Reference | Description | Use |
| --- | --- | --- |
| GatewayV2 Software Requirements Specification | Normative firmware behavior, state machines, timeout/retry rules and verification criteria | Firmware / QA |
| Ethernet Wifi Gateway schematic package | Current 12-page hardware schematic, PCB view and BOM used as the hardware basis for Rev 1.1 | Hardware / Product |
| README.md | Project overview, build and bring-up entry point | Engineering |
| README_STRUCTURE.md | Current software architecture and repository organization | Engineering |

## Approval

| Role | Name | Status | Date |
| --- | --- | --- | --- |
| Product owner | TBD | Pending | TBD |
| System integration | TBD | Pending | TBD |
| Firmware | TBD | Pending | TBD |
| Hardware | TBD | Pending | TBD |
| QA / Verification | TBD | Pending | TBD |

# Contents

Document structure and section navigation

1. Product Overview	5

2. Key Features	6

3. Hardware Platform	7

3.1 MCU and firmware platform	7

3.2 Non-volatile storage	7

3.3 Ethernet	7

3.4 Wi-Fi and Bluetooth module	8

3.5 LTE modem	8

3.6 RS485	8

3.7 USB	9

3.8 Power architecture and electrical characteristics	9

3.9 External connectors and service interfaces	10

3.10 Mechanical characteristics	10

3.11 Environmental characteristics	10

3.12 Protection and compliance	11

4. Network	12

4.1 Ethernet	12

4.2 Wi-Fi	12

4.3 LTE	12

4.4 Network failover	12

5. Industrial Communication	13

5.1 Modbus RTU	13

5.2 Modbus TCP	13

6. Cloud Communication	14

6.1 MQTT	14

6.2 NTP	14

7. Web Configuration	15

7.1 Access	15

7.2 HTTP services	15

7.3 Dashboard information	15

8. Firmware and Storage	16

8.1 Configuration	16

8.2 Firmware lifecycle and update	16

8.3 Factory reset and reboot	16

9. Diagnostics	17

10. Security	18

11. Performance and Reliability	19

12. Build and Firmware Baseline	20

13. Current Limitations	21

14. Future Features	22

15. Release Readiness and Open Decisions	23

# 1. Product Overview

GatewayV2 is an embedded industrial IoT gateway for collecting data from RS485/Modbus field devices and bridging it to local Ethernet networks, Wi-Fi, LTE and cloud services.

The product provides:

Three independent RS485/Modbus RTU channels.

Modbus TCP access for local SCADA or supervisory systems.

MQTT telemetry transport to a configured broker.

Automatic Ethernet, Wi-Fi and LTE route failover.

Local HTTP Web Config and diagnostics.

Optional mDNS-based Web Config discovery when multicast is available.

Persistent configuration and NTP time synchronization.

# 2. Key Features

| Feature | Specification | Status | Notes |
| --- | --- | --- | --- |
| Field protocol | Modbus RTU over RS485 | Supported | Three independent channels |
| Local protocol | Modbus TCP | Supported | Default service port 502 |
| Cloud protocol | MQTT over TCP/IP | Supported | Broker and topic configurable |
| Network connectivity | Ethernet, Wi-Fi, LTE | Supported | Automatic route selection and failover |
| Web Config | HTTP on port 80 | Supported | Local network access |
| Web discovery | mDNS synaptix-gateway.local | Supported when available | Requires usable LAN and multicast support |
| Time | NTP synchronization | Supported | Requires a usable IP route |
| Diagnostics | Serial log, shell and HTTP status | Supported | Runtime network and service status |
| USB networking | RNDIS/ECM Web Config | Deferred | Disabled in current baseline |

# 3. Hardware Platform

Hardware information in this section is based on the current GatewayV2 schematic/BOM. Component presence and circuit topology are considered the current engineering baseline; electrical limits, environmental ratings and compliance claims remain subject to released hardware documentation and qualification.

## 3.1 MCU and firmware platform

| Item | Specification | Notes |
| --- | --- | --- |
| MCU | STM32H563VGT6 | LQFP100 |
| RTOS | FreeRTOS | Current firmware baseline |
| IP stack | lwIP | Current firmware baseline |
| Main MCU clock source | 16 MHz crystal | Board hardware |
| Low-speed clock source | 32.768 kHz crystal | RTC/low-speed domain |
| Debug/programming | SWD available on engineering header | Internal engineering interface |
| Application architecture | Embedded multi-task gateway firmware | Firmware baseline |
| Primary build output | GatewayV2.bin | Internal build artifact |

## 3.2 Non-volatile storage

| Item | Specification | Notes |
| --- | --- | --- |
| SPI flash | W25Q128 | 128 Mbit device; current persistent-storage path |
| OctoSPI flash | MX25LM51245GXDI00 | High-capacity external memory fitted in current schematic |
| Configuration storage | Persistent | Validated before runtime use |
| Storage allocation | Product partition/allocation contract TBD | Must be aligned with firmware/OTA design before release |

## 3.3 Ethernet

| Item | Specification | Notes |
| --- | --- | --- |
| Interface | 10/100 Ethernet | Product network interface |
| PHY | DP83848 | RMII connection to MCU |
| Ethernet clock | 50 MHz oscillator | PHY reference clock |
| Connector | HR911105A RJ45 | Integrated magnetics/LED interface in current design |
| Link indication | RJ45 link/activity LEDs | Hardware provisioned |
| Addressing | DHCP or static IPv4 | Firmware configurable |
| Web Config | Available on usable Ethernet IP | Current application |
| Default route | Supported | Managed by Network Manager |

## 3.4 Wi-Fi and Bluetooth module

| Item | Specification | Notes |
| --- | --- | --- |
| Module | ST67W611M1A6UTR | Wi-Fi 6 / Bluetooth LE combo coprocessor hardware |
| Product Wi-Fi mode | Station | Current GatewayV2 application baseline |
| MCU interface | SPI plus control/ready signals | UART/debug signals also routed in hardware |
| Low-speed reference | 32.768 kHz crystal | Module hardware |
| RF connector | SMA-KWE antenna connector | One Wi-Fi antenna port in current design |
| Addressing | DHCP through configured access point | Current firmware |
| Web Config | Available on usable Wi-Fi IP | Current application |
| Default route | Supported | Managed by Network Manager |
| Bluetooth LE | Hardware capability present | Not exposed by current GatewayV2 product application |

## 3.5 LTE modem

| Item | Specification | Notes |
| --- | --- | --- |
| Modem | A7680C | Cellular modem module |
| Host data interface | UART / PPPoS | 1.8 V / 3.3 V interface adaptation present in hardware |
| SIM | Nano-SIM socket, 7-pin | SIM ESD protection fitted |
| RF connector | SMA-KWE antenna connector | One LTE antenna port in current design |
| Hardware indicators | STATUS and NETLIGHT | LED driver circuitry fitted |
| Control | PWRKEY and RESET | Driven from MCU through transistor interface circuitry |
| Default route | Supported as cellular fallback route | Current application |
| Web Config | Not directly advertised as a LAN access path | Current product behavior |

## 3.6 RS485

| Item | Specification | Notes |
| --- | --- | --- |
| Number of channels | 3 independent channels | RS485-1 / RS485-2 / RS485-3 |
| Transceiver | 3 x MAX485 | Half-duplex interface with independent DE/RE control |
| Protocol | Modbus RTU | Current product application |
| Termination | 120 ohm per channel | Fitted in current schematic |
| Line filtering | ACM2520-2P common-mode choke per channel | Hardware provisioned |
| Line protection | 9.2 V TVS network per channel | Circuit-level transient protection; qualification TBD |
| Connector | 2-pin, 5.08 mm terminal block per channel | CN2, CN3, CN5 |
| Configuration | Baud rate, parity, stop bits, address and timeout | Firmware configurable |
| Channel architecture | Independent channels | Separate UART and DE/RE control |
| Fault behavior | Failure on one channel does not stop other channels or network services | Product behavior |
| Galvanic isolation | Not implemented in the current schematic | Product decision required if field isolation is required |

| NOTE | Hardware review note: The RS485 transceiver supply-selection population between 3.3 V and 5 V shall be confirmed against the released assembly/BOM before production approval. |
| --- | --- |

## 3.7 USB

| Item | Current release / hardware | Notes |
| --- | --- | --- |
| Connector | USB Type-C, 16-pin receptacle | Hardware fitted |
| Data protection | USBLC6-2SC6 ESD protection | Circuit-level protection |
| EMI filtering | ACM2520-2P common-mode choke | USB D+/D- path |
| VBUS protection | 1 A / 5 V fuse and SS34 series diode | Hardware provisioned |
| USB network interface | Disabled | Firmware baseline |
| USB Web Config URL | Not available | Deferred feature |
| Internet failover participation | Not applicable | USB excluded from route selection |
| Planned use | Local maintenance/configuration network | Future release |

## 3.8 Power architecture and electrical characteristics

| Item | Specification | Status / Notes |
| --- | --- | --- |
| Nominal input | 24 VDC | Current schematic input rail |
| Input connector | 2-pin, 5.08 mm terminal block | CN1 |
| Input fuse | 1 A / 24 V | Current hardware |
| Reverse-polarity path | Series SS34 Schottky diode | Current hardware; system-level reverse-polarity limits TBD |
| Input transient clamp | 24 V TVS diode | Current hardware; surge immunity rating not yet qualified |
| DC/DC conversion | 2 x TPS54302 buck regulators | Generates board 3.3 V and 5 V rails |
| Internal rails | 3.3 V and 5 V | Current hardware |
| Qualified input voltage range | TBD | Requires electrical qualification; do not infer from nominal 24 V input |
| Typical power consumption | TBD | Pending measurement on release hardware |
| Maximum power consumption | TBD | Pending worst-case measurement |
| Power-up / brownout behavior | TBD | Requires system qualification and firmware acceptance criteria |

## 3.9 External connectors and service interfaces

| Interface | Hardware | Product exposure |
| --- | --- | --- |
| Power | 2-pin 5.08 mm terminal block | External |
| RS485 x3 | 2-pin 5.08 mm terminal blocks | External |
| Ethernet | HR911105A RJ45 | External |
| USB | USB Type-C receptacle | External hardware; network function disabled in current firmware |
| LTE SIM | Nano-SIM socket | External/service access depending on enclosure |
| LTE antenna | SMA-KWE | External RF connector |
| Wi-Fi antenna | SMA-KWE | External RF connector |
| MCU SWD / debug headers | Engineering headers | Internal engineering/service only |
| ST67W debug header | Engineering header | Internal engineering/service only |
| Status indicators | 3.3 V, 5 V, LTE STATUS/NETLIGHT and MCU/user LEDs | Enclosure exposure TBD |
| Local buttons | MCU reset/boot and modem control circuitry | Enclosure exposure TBD |

## 3.10 Mechanical characteristics

| Item | Specification | Status |
| --- | --- | --- |
| PCB/enclosure dimensions | TBD | Pending mechanical design release |
| Weight | TBD | Pending product measurement |
| Mounting | TBD | Pending enclosure/mechanical definition |
| Enclosure | TBD | Pending mechanical design release |
| Connector cutouts / accessibility | TBD | Must match released enclosure |

## 3.11 Environmental characteristics

| Item | Specification | Status |
| --- | --- | --- |
| Operating temperature | TBD | Pending environmental qualification |
| Storage temperature | TBD | Pending environmental qualification |
| Relative humidity | TBD | Pending environmental qualification |
| Vibration/shock | TBD | Pending product qualification |
| Installation altitude | TBD | Pending product qualification |

## 3.12 Protection and compliance

| Area | Current hardware provision | Qualification / product status |
| --- | --- | --- |
| 24 V input | 1 A fuse, SS34 series diode, 24 V TVS | Electrical limits and surge qualification TBD |
| USB | USBLC6-2SC6 ESD device and common-mode choke | Certified ESD/EMC immunity TBD |
| SIM interface | ESDA6V1-5SC6 protection array | Certified ESD immunity TBD |
| RS485 | 9.2 V TVS network and common-mode chokes | Surge/ESD immunity TBD |
| Galvanic isolation | No galvanic isolation implemented on current RS485 schematic | Product requirement decision required |
| EMC emissions/immunity | Protection/filter components present | Formal EMC test pending |
| CE/FCC or regional marking | Not yet claimed | Compliance plan TBD |

Circuit-level protection components listed above describe the current hardware design only. They shall not be interpreted as certified ESD, surge, safety or EMC performance until qualification testing is complete.

# 4. Network

## 4.1 Ethernet

Ethernet supports DHCP or static IPv4 configuration. When link is stable and the interface has a valid IPv4 address, it may be selected as the default route and may serve Web Config.

## 4.2 Wi-Fi

Wi-Fi operates in station mode and connects to the configured access point. After association and DHCP completion, it may serve Web Config and become the default route.

## 4.3 LTE

LTE uses the A7680C modem and PPPoS to provide a cellular IP connection. The SIM must report ready, the modem must register with the operator and packet attach must succeed before LTE is usable.

## 4.4 Network failover

| Property | Specification | Classification |
| --- | --- | --- |
| Interface operation | Ethernet, Wi-Fi and LTE may initialize independently | Product capability |
| Default route | One active route at a time | Product capability |
| Selection | Automatic based on interface usability and configured priority | Product capability |
| Usable state | Stable link, interface UP and valid IPv4 address | Product definition |
| Failover | Automatic; reboot not required | Product capability |
| Ethernet debounce | 1 second | Current baseline; verify before release |
| Route-change debounce for cloud reconnect | 500 ms | Current baseline; verify before release |
| USB participation | Disabled and excluded from route selection | Release configuration |

When the default route changes, cloud services reconnect through the new usable route. Existing IP sessions may be interrupted during transition.

# 5. Industrial Communication

## 5.1 Modbus RTU

GatewayV2 supports three concurrent RS485/Modbus RTU channels. Each channel can be configured for the connected field devices and can expose mapped register data to the application.

Supported configuration concepts include:

Serial baud rate.

Data parity and stop bits.

Device/slave address.

Request and response timeout.

Register address and data type.

Scaling and offset for application values.

## 5.2 Modbus TCP

| Item | Specification |
| --- | --- |
| Service | Modbus TCP server |
| Default port | 502 |
| Data source | Configured Modbus RTU devices and mapped gateway data |
| Availability | Any usable IP interface that permits local access |

# 6. Cloud Communication

## 6.1 MQTT

| Item | Specification | Classification / Notes |
| --- | --- | --- |
| Protocol | MQTT | Product capability |
| Transport | TCP/IP | Product capability |
| Default port | 1883 | Current baseline |
| Broker | Configurable | Product capability |
| Topic base | Configurable | Product capability |
| Network interfaces | Ethernet, Wi-Fi, LTE | Product capability |
| DNS | Supported | Product capability |
| Automatic reconnect | Supported | Product capability |
| Route failover | Supported | Product capability |
| Retry backoff | Supported | Product behavior |
| Maximum retry delay | 60 seconds | Current baseline; verify before release |
| QoS | TBD — exact product-level QoS contract not yet finalized | Product decision required |
| Offline queue | TBD — exact product-level offline buffering contract not yet finalized | Product decision required |

MQTT is an application-level service and does not require a specific physical network interface. It follows the currently selected default route.

Detailed MQTT state-machine, timeout, retry and callback behavior is defined in the Software Requirements Specification.

## 6.2 NTP

| Item | Specification |
| --- | --- |
| Service | Network Time Protocol |
| Purpose | Synchronize system time |
| Requirement | A usable IP route and reachable time server |
| Diagnostics | Synchronization result and Unix timestamp are logged |

# 7. Web Configuration

## 7.1 Access

| Access method | URL | Availability |
| --- | --- | --- |
| mDNS | http://synaptix-gateway.local/ | Usable LAN plus multicast support |
| Ethernet | http://<ETH_IP>/ | Ethernet has valid IPv4 |
| Wi-Fi | http://<WIFI_IP>/ | Wi-Fi has valid IPv4 |
| USB | http://192.168.7.1/ | Deferred; USB disabled |

The IP address shown by the gateway is authoritative when mDNS is unavailable. The product does not require a fixed Ethernet or Wi-Fi IP address.

## 7.2 HTTP services

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/status | Device, network and service status |
| GET | /api/network/discovery | Available Web Config URLs |
| GET | /api/config | Current configuration |
| POST | /api/config | Validate and save configuration |
| POST | /api/reboot | Controlled device reboot |

## 7.3 Dashboard information

The dashboard displays device identity, firmware version, uptime, active route, interface addresses, MQTT state, resource usage, field-device status and available Web Config access URLs.

# 8. Firmware and Storage

## 8.1 Configuration

| Configuration property | Specification |
| --- | --- |
| Persistence | Retained across reboot |
| Validation | Performed before activation |
| Invalid update | Rejected |
| Last valid configuration | Preserved |

## 8.2 Firmware lifecycle and update

| Feature | Current release |
| --- | --- |
| Local firmware image generation | Supported |
| Standard flashing flow | Supported |
| Secure remote OTA | Not implemented |
| Image signing | Not implemented |
| Anti-rollback | Not implemented |

## 8.3 Factory reset and reboot

Controlled reboot is available through the supported management interface. Factory-reset behavior and production recovery policy require a product-level decision before certification.

# 9. Diagnostics

| Diagnostic area | Information provided |
| --- | --- |
| Boot | Reset flags, initialization status and firmware startup |
| Network | Link, administrative state, IP, gateway, active route and failover |
| MQTT | Connected state, broker, route, retry and error status |
| DNS | Resolution result and failure status |
| LTE | SIM, signal, operator, registration, packet attach and PPP status |
| Wi-Fi | Module startup, association, link and IP status |
| Modbus | Channel and device communication status |
| Resources | CPU, heap, lowest free heap and task stack usage |
| Serial output | Serialized system, vendor/NCP, shell and diagnostic frames |

# 10. Security

| Security feature | Current release | Product direction |
| --- | --- | --- |
| Local HTTP Web Config | Supported on trusted LAN | Retain for commissioning |
| Web authentication | Not supported | Planned |
| HTTPS | Not supported | Planned |
| MQTT TLS | Not supported/TBD | Security review required |
| Secure credential storage | Not finalized | Security review required |
| Signed OTA | Not supported | Planned |
| Anti-rollback | Not supported | Planned |

The current release must be deployed on a trusted management network. It must not be marketed as providing secure remote administration until the planned security features are implemented and verified.

# 11. Performance and Reliability

| Property | Specification | Classification |
| --- | --- | --- |
| MQTT reconnect | Starts within 3 s after route is stable and has valid IP | Engineering target; verify before release |
| Parallel MQTT attempts | Maximum 1 | Product behavior |
| Active DNS requests for MQTT | Maximum 1 | Product behavior |
| Route transition | No reboot required | Product capability |
| Interface failure | Unrelated interfaces and services remain operational | Product capability |
| UART logging | Complete, ordered frames under concurrent output | Product capability |
| Free heap | At least 20% under defined product workload | Engineering target; verify on release hardware |
| Modbus channel independence | One channel failure does not stop other channels | Product capability |

The timing, retry and resource values in this section are release targets or observed engineering behavior only until product management and qualification tests formally approve them. Detailed state-machine and timeout behavior is defined in the Software Requirements Specification.

# 12. Build and Firmware Baseline

This section is included because this document is classified as Internal Engineering. It is not part of an external customer datasheet.

| Item | Baseline |
| --- | --- |
| Product image | GatewayV2.bin |
| Standard output directory | build |
| Build system | CMake with approved ARM toolchain |
| USB networking | OFF |
| mDNS | ON when multicast support is enabled |
| HTTP port | 80 |
| Modbus TCP port | 502 |
| MQTT default port | 1883 |

Production builds shall use the documented build and flash scripts and shall not be confused with temporary USB-on, USB-off or clean-build test variants.

# 13. Current Limitations

mDNS depends on host operating-system support, multicast delivery and local network policy; the interface IP URL remains the fallback.

A fixed hostname can conflict if multiple gateways use the same hostname on one LAN; fleet identity and conflict management are not yet defined.

Web Config is HTTP-only and has no authentication in the current release.

USB networking is not available in the current firmware baseline even though USB Type-C hardware is fitted.

LTE registration and signal quality depend on antenna, SIM, operator coverage and modem power conditions.

MQTT TLS, signed OTA and secure credential storage are not yet part of the released security model.

Qualified input-voltage range, power consumption, mechanical dimensions and environmental limits remain TBD.

The current RS485 implementation is not galvanically isolated; suitability for field installation shall be confirmed at product level.

Circuit-level TVS/ESD/filter components are present, but no EMC/surge/ESD compliance claim is made until formal qualification.

# 14. Future Features

USB RNDIS/ECM local configuration network.

HTTPS and authenticated Web Config.

MQTT TLS and protected credential provisioning.

Signed OTA with anti-rollback.

Unique per-device discovery identity and multi-gateway provisioning.

Fleet monitoring and cloud device registry.

Expanded production, environmental and EMC qualification data.

# 15. Release Readiness and Open Decisions

The following items must be resolved or formally accepted before this specification can be promoted from draft to an approved released baseline:

| Area | Open item | Required source / decision |
| --- | --- | --- |
| Hardware power | Qualified 24 V input range and worst-case power consumption | Electrical measurement and hardware qualification |
| RS485 | Confirm transceiver supply population and decide whether galvanic isolation is required | Released BOM/assembly data and product/system decision |
| Mechanical | PCB/enclosure dimensions, mounting, connector access and weight | Mechanical release |
| Environment | Temperature, humidity, vibration/shock and altitude | Qualification plan and test report |
| Compliance | ESD, surge and EMC performance; regional marking | Compliance plan and qualification results |
| MQTT | QoS and offline queue product contract | Product/system decision |
| Security | Web authentication, HTTPS, MQTT TLS and credential storage | Security architecture decision |
| Firmware update | Signed OTA and anti-rollback policy | Product/security decision |
| Device identity | Unique hostname/provisioning for multi-gateway LAN | Product/system decision |
| Recovery | Factory-reset and production recovery behavior | Product/system decision |
| Performance | Reconnect, heap and timing targets | Release-hardware verification |

END OF DOCUMENT  •  GWV2-PTS-001